Mega Pacific Technology

# TECHNICAL BULLETIN

## MEGA DS-11 De-size enzyme

Description:

DS-11 is an alpha amylase, which can decompose the amylum sizing quickly during in de-size application with fabric or garment.

### Specification:

Amber liquid enzyme with enzyme smell.

Activity: 2000u/ml

pH: pH range 5.5-7.5

Application temperature:

DS-11 especially for textile de-size purpose, available for wide range temperature from 35c to 95c.

Dosage:

DS-11 can decompose the amylum sizing without any unfavorable effects on the fabric. The de-sizing conditions, the composition of amylum sizing and the type of fabric determine the optimum dosage of amylase.  DS-11 amylase can generally work well with the dosage between 1g/l to 3g/l.

DS-11 can be used at any kind of laundry machine.

DS-11 is compatible with any nonionic wetting agent.

Standard Packing:

200 kgs drum or one ton toe.

Storage: If keep on 25c, the sift life is min. 3 months. Avoid to keep higher then 30c. for long time.

Safety:

DS-11 is an un- flammable liquid, easy to in water soluble. Save to be used. Avoid contact DS-11 directly.  Any accidence contacts the skin or eyes with DS-11, promptly wash with clean water.